# Edge-Computing Video Analytics for Real-Time Traffic Monitoring in a Smart City

## Case Study Analysis: Liverpool Smart Pedestrians Project

A. Introduction and Objectives
The Liverpool Smart Pedestrians project aimed to develop a real-time pedestrian monitoring system using edge computing and video analytics. The main goal was to collect accurate pedestrian flow data while also preserving privacy and reducing network load. This project addressed urban planning challenges such as overcrowding, pedestrian safety, emergency evacuation planning, and lack of real-time pedestrian data.

B. Methodology
The project used an edge-computing approach where video processing and analytics were performed directly on a smart sensor device. The researchers designed and evaluated an edge device capable of detecting and tracking pedestrians in real time. Key constraints included limited processing power, energy efficiency, real-time response, privacy protection, and outdoor operational conditions.

C. Technology and Implementation
The system was built using the NVIDIA Jetson TX2 platform, selected for its strong GPU performance and low power consumption. YOLO V3 was used for pedestrian detection due to its real-time capabilities and accuracy. PyTorch supported model development, and the SORT algorithm enabled efficient tracking. Edge computing reduced latency and bandwidth use, aligning with smart city principles discussed in Module 2.

D. Validation and Performance
Validation experiments measured detection accuracy, processing speed, and system resource usage. Results showed that the system achieved real-time performance with acceptable accuracy while efficiently using GPU and CPU resources. These experiments confirmed the suitability of edge devices for continuous urban monitoring.

E. Real-World Applications
Two real-world deployments were tested. An indoor emergency evacuation scenario showed the system’s ability to monitor crowd movement and identify congestion points. The outdoor deployment in Liverpool collected pedestrian flow data in public spaces, supporting urban planning decisions. These applications align with Module 3 concepts involving both, the sensor technologies and edge gateways as well.

F. Challenges and Future Work
Challenges included hardware limitations, environmental variability, and maintaining accuracy in dense crowds. Future work suggested optimizing AI models and improving scalability. Since 2019, advancements such as YOLOv5/YOLOv8, NVIDIA Jetson Xavier and Orin platforms, 5G connectivity, and improved edge AI frameworks could enhance system performance.

G. Personal Evaluation
The project successfully demonstrated an effective edge-based smart city solution. It balanced performance, privacy, and scalability. Future improvements could include multi-sensor integration, enhanced robustness to weather conditions, and real-time city dashboard integration. Overall, this was a great learning experience, and I was able to come out with a greater knowledge about edge computing video analytics than I had before.

References
Barthélemy, J., Verstaevel, N., Forehead, H., & Perez, P. (2019). Edge-computing video analytics for real-time traffic monitoring in a smart city. Sensors, 19(9), 2048. https://doi.org/10.3390/s19092048